AI-BASED SMART TRAFFIC MANAGEMENT

Methodology, Novelty and Key Contributions

Department of Computer Science and Engineering, TKR College of Engineering & Technology

Batch A10 (CSE-A)  ·  Academic year [ACADEMIC YEAR]

Table 1: Project team

| S.No. | Student name | Roll number |
| --- | --- | --- |
| 1 | Buneti Harshitha | 23K91A0548 |
| 2 | Bhuvaneshwari Kadike | 23K91A0537 |
| 3 | Ambati Sai Nikhil Reddy | 23K91A0510 |
| 4 | Chapala Rohith Reddy | 23K91A0551 |

Project guide: Laxmi Prasanna

Chapter and section numbers quoted in the text (for example Chapter 8 or Section 6.2) refer to the full project report, 17.Final Document. Figures and tables in this document are numbered from 1.

# 1. METHODOLOGY

This document describes how FlowSense works from end to end and what is new in it. Part 1 gives the pipeline, the data, the algorithms and the reasons they were chosen; Part 2 sets the work against the base paper and separates what is new from what is standard; Part 3 lists the contributions.

## 1.1 Overall methodology

FlowSense treats traffic management as one measurable pipeline. Vehicles are counted and classified from a camera frame, queues are forecast a short time ahead, signal timing is computed from the measured and forecast demand, an ambulance is given a green corridor, neighbouring junctions share what they can see coming, and violations are logged. The base paper [1] supplies one stage of this chain, emergency pre-emption triggered by a light-bar detector, on top of an optimised controller; FlowSense keeps that idea and builds the other stages around it. Because a real junction cannot be used for experiments, every stage that touches signals is evaluated in a seeded traffic-network simulator that runs four controllers on identical traffic. The method has seven steps:

1.  Generate demand. Vehicles arrive at the entry roads of a 2 x 2 or 3 x 3 grid of junctions as Poisson arrivals, scaled by a demand slider and a morning and evening rush-hour profile. A fixed seed makes the demand identical for all controllers.

2.  Sense. At every second each approach knows its queue (vehicles and passenger-car units), the vehicles that will arrive from upstream within 8 s, and whether an ambulance is queued or is within 15 s.

3.  Forecast. Every 10 s a gradient-boosted model predicts each approach's queue 30 s ahead from eleven features.

4.  Decide. Each junction compares the pressure of its green phase with that of the red phase, and switches under minimum and maximum green rules; an ambulance overrides the comparison.

5.  Discharge and move. Queued vehicles leave at the saturation flow while their phase is green, travel the 12 s link to the next junction or leave the network; a full downstream queue blocks the release (spill-back).

6.  Observe. The simulator records waiting time, queues, throughput, ambulance delay, violations, idling fuel and CO2; the vision layer in parallel detects vehicles in an image, flags light bars and recommends a green time.

7.  Compare. Five seeds per setting, two grid sizes, a demand sweep, an ambulance-heavy test and an ablation are run by scripts/run_experiments.py, and the vision layer is scored on synthetic scenes and on two public sets of real photographs.

Figure 1: Methodology as a pipeline: demand, sensing, forecasting, decision, discharge and observation.

## 1.2 Data

FlowSense has three kinds of data, and keeping them apart matters for reading the results. Simulated traffic is the only input to the controllers: it is generated, not collected, and its parameters (arrival rates, saturation flow, link time, vehicle mix) are the project's own assumptions. Training data for the forecaster is generated by the same simulator under the adaptive controller. Real photographs are used only to evaluate the vision layer; nothing is trained on them.

Table 2: Datasets and their roles.

| Data | Source | Size | Role |
| --- | --- | --- | --- |
| Simulated demand | Sim with seeds 40-44 (benchmarks) | 5 seeds x 1,800 s per setting | Controller comparison |
| Forecaster training set | Six simulated runs, adaptive controller, snapshots every 10 s | 26,988 rows x 11 features | Train and test the queue forecaster |
| Synthetic overhead scenes | render_scene, seeds 1000-1299 | 300 scenes, 2,754 vehicles | Detector check (in-domain) |
| Real mixed-traffic frames | Visaitech v0.1 (CC BY 4.0) [32] | 293 frames, 1,506 boxes (1,467 vehicles) | Evaluate YOLOv8n |
| Real ambulance photographs | shravya11 (CC BY 4.0) [33] | 300 ambulance + 300 other photos (of 4,749) | Evaluate the light-bar rule |
| Bundled sample photograph | Ultralytics bus.jpg | 1 image, 1080 x 810 | Vision-page demonstration |

The simulator's demand model is a Poisson process per entry road with base rates of 0.15 vehicles per second on the north and south entries and 0.10 on the east and west entries, multiplied by the demand slider and by a time-of-day profile that peaks at 08:30 and 17:30 (the equation below). Vehicles are 68 % cars, 13 % bikes, 6 % buses and 13 % trucks; 5 % of cars are red-light runners that cross after waiting 25 s on a red, and a wrong-way driver is generated with probability 0.0003 per entry road per second. Each vehicle has a route drawn once: straight with probability 0.78, a turn to one side with 0.11 and the other with 0.11; ambulances always go straight. Fuel and CO2 are derived from queue length (the fuel equation in Section 6.5.4), not simulated per vehicle.

λ_a(t) = r_a · d · m(h),   m(h) = 0.7 + 0.5·exp(−((h − 8.5)/0.9)²) + 0.55·exp(−((h − 17.5)/1.1)²),   N_a(t) ~ Poisson(λ_a(t))      (1)

Three limits of this data are stated here because later chapters depend on them. The demand and discharge parameters are not calibrated to a real junction. The real photographs come from a dashcam in South Asian mixed traffic (2-wheelers, auto-rickshaws) and from general ambulance photographs, not from fixed junction cameras. And the two real sets were used with the application's own default settings, with no tuning on them.

## 1.3 Feature construction

There is no image preprocessing for the controllers; the simulator hands them counts. Feature construction matters only for the forecaster. For each approach Sim.feature_matrix() returns one row of eleven numbers, computed from the queue history, the arrivals and the signal state. The target is the approach's queue length 30 s later. No scaling is applied because the model is tree-based.

Table 3: Forecaster features.

| Feature | Definition | GB importance |
| --- | --- | --- |
| queue | Vehicles waiting now | 0.692 |
| pcu_queue | The same queue weighted by passenger-car units | 0.088 |
| queue_lag10 | Queue 10 s ago | 0.008 |
| queue_lag30 | Queue 30 s ago | 0.133 |
| inflow_30s | Vehicles that arrived in the last 30 s | 0.001 |
| inbound_15s | Vehicles in transit due within 15 s | 0.004 |
| is_green | 1 if this approach's phase is green and not clearing | 0.022 |
| green_time | Seconds since the junction's phase began | 0.005 |
| is_entry | 1 if the approach is fed from outside the grid | 0.004 |
| sin_day | sin(2π·hour/24) | 0.027 |
| cos_day | cos(2π·hour/24) | 0.016 |

The current queue carries 69.2 % of the importance, the queue 30 s ago 13.3 % and the PCU queue 8.8 %: the forecaster is mostly a refined "the queue stays about what it is" model, which is why it improves on persistence by 35 % and not by more. The 30-second inflow and the inbound count carry almost no importance, so the forecaster uses little information about arriving platoons; the controller adds that information separately through its inbound term.

## 1.4 Algorithm choice and justification

### 1.4.1 Pressure-based signal control

Fixed-time plans of the Webster kind [25] are what most junctions run; they are the baseline. A queue-actuated rule that compares the demand of the two phases is simple, needs only counts, and is the family that max-pressure control [26] belongs to. Reinforcement-learning controllers [27] can do better but need long training in a simulator and are hard to inspect, so the project preferred a rule whose every term can be explained and ablated. The advanced controller keeps the adaptive rule's switching test and changes only what it calls demand: the PCU-weighted queue plus a forecast term, an arriving-platoon term, a fairness term and a spill-back damping factor.

### 1.4.2 Passenger-car units

A bus occupies more road and discharges more slowly than a bike, so counting vehicles misjudges demand. Passenger-car-equivalent weights are standard practice [31]; FlowSense uses car 1, bike 0.5, truck 2, bus 3 and ambulance 1, and uses the same weights in the simulator's pressure and in the vision layer's density estimate so that a camera count could feed the controller directly.

### 1.4.3 Forecast term, inter-junction look-ahead and the green wave

A queue forecast lets the controller give green to an approach before its queue has grown; the term is 0.7 · max(forecast − queue, 0), so a forecast below the present queue never reduces demand and a poor forecast can only add a bounded bias. Look-ahead is the project's reading of "inter-junction communication": a junction reads the transit list of vehicles already released by its neighbours and adds their PCU to the pressure of the approach they will join. A green wave follows from extending the emergency look-ahead from 5 s (one junction, base-paper style) to 15 s, so that junctions along the ambulance's route switch before it arrives. Spill-back damping multiplies pressure by 0.3 when the receiving queue is within 4 vehicles of capacity, so that green is not wasted on vehicles that cannot move.

### 1.4.4 Gradient boosting for the forecaster

Three models were compared: persistence, Random Forest [29] and gradient boosting [28]. On the application's own split the two ensembles are equivalent (MAE 1.605 for Random Forest and 1.644 for gradient boosting; persistence 2.535). Gradient boosting was kept because it was already the shipped model and is the smaller of the two, not because it won; a ridge regression was also tried and is only slightly better than persistence on random splits. Chapter 8 reports how all of them behave on a simulation they have not seen.

### 1.4.5 Pretrained YOLOv8n and an HSV light-bar rule

YOLO [21] detectors give counts and classes in one pass; the nano variant of YOLOv8 [22] runs in tens of milliseconds on a CPU, and its COCO weights already cover car, motorcycle, bus and truck [23]. The base paper trained a YOLO26m light-bar detector on 3,130 annotated images; the batch had no such dataset or GPU, so FlowSense uses a hand-written rule instead: red and blue pixels in fixed HSV ranges, dilated, whose intersection marks a light bar. The rule is transparent and needs no data, and its weakness is exactly that it is hand-tuned; Chapter 8 measures how far it falls from a trained detector.

### 1.4.6 Why a simulator, and why not SUMO

SUMO [24] is the accepted tool and the one the base paper used, but it needs installation, a network and a route file for every experiment and a TraCI client for control. A 415-line Python simulator runs 1,800 s in a fraction of a second, makes the identical-demand guarantee trivial, and can be shipped inside the web application. The price is that the simulator's absolute numbers are the project's, and Chapter 8 compares percentages with the base paper, never seconds.

## 1.5 Mathematical formulation

Every formula below is what the code computes. Symbols: j a junction, a an approach (N, E, S, W), φ a phase (0 north-south, 1 east-west), Q the queue (a list of vehicles), π the PCU weight, CAP = 30 the approach capacity, t the simulated second.

### 1.5.1 Queue weight and the advanced pressure

W_{j,a} = Σ_{v ∈ Q_{j,a}} π(type(v)),   π(car) = 1, π(bike) = 0.5, π(bus) = 3, π(truck) = 2, π(ambulance) = 1      (2)

P_{j,a} = ( W_{j,a} + 0.7·max(f̂_{j,a} − |Q_{j,a}|, 0) + I_{j,a} + w_head/12 ) · γ_{j,a},   γ = 0.3 if |Q_down(j,a)| ≥ CAP − 4 else 1      (3)

Here f̂ is the forecast queue, I the PCU of vehicles inbound within 8 s, w_head the waiting time in seconds of the vehicle at the head of the queue (0 if empty), and Q_down the queue of the receiving approach straight ahead. The adaptive and emergency-aware controllers use P = W only. The phase pressure is the sum over its two approaches:

Π_{j,φ} = Σ_{a ∈ φ} P_{j,a}      (4)

### 1.5.2 Switching rule

With g_min = 15, g_max = 60 for the adaptive and emergency-aware controllers and g_min = 13, g_max = 60 for the advanced one, a junction in phase φ with green age τ switches to the other phase φ' (after a 3 s clearance) when

τ ≥ g_min  and  ( Π_{j,φ} ≤ 0.01 and Π_{j,φ'} > 0,   or   Π_{j,φ'} > 2·Π_{j,φ} + 4,   or   τ ≥ g_max and Π_{j,φ'} > 0 )      (5)

The fixed-time controller instead switches when τ ≥ 30. After a phase begins, vehicles are released only after a 2 s start-up loss, at saturation flow s = 0.5 vehicles per second per approach: a credit accumulates by s every green second and one vehicle leaves per whole credit.

credit ← credit + s each green second after τ > 2;   release the head vehicle while credit ≥ 1 and the next queue has fewer than CAP vehicles      (6)

### 1.5.3 Emergency target

For a junction, let d_a be 0 if an ambulance is in the queue of approach a, otherwise the number of seconds until an inbound ambulance arrives (only if within the look-ahead L, 5 s for the emergency-aware controller and 15 s for the advanced one). Then

a* = argmin_a d_a,   φ* = phase(a*);   if φ* ≠ φ and τ ≥ 2 then switch to φ* now, else hold φ      (7)

Inside the discharge step an ambulance is moved to the head of its queue ("sirens clear the lane"). The count of pre-emptions increments once per continuous override.

### 1.5.4 Forecast, density and resources

ŷ_{j,a} = clip( GB(x_{j,a}(t)), 0, 40 ),   y_{j,a} = |Q_{j,a}(t + 30)|,   MAE = (1/n) Σ |y − ŷ|,   R² = 1 − Σ(y − ŷ)² / Σ(y − ȳ)²      (8)

GB: F_m(x) = F_{m−1}(x) + ν·h_m(x),   ν = 0.08,   m = 1 … 160,   h_m a regression tree of depth ≤ 4 fitted to the residuals on a 0.9 subsample      (9)

ρ = Σ_c π(c)·n_c / 14,   level = LOW (ρ < 0.35), MEDIUM (< 0.7), HIGH (< 1.0), JAMMED;   g = 12 if an ambulance is present else min(60, max(15, 12 + 3.2·Σ_c π(c)·n_c))      (10)

Fuel (L) = ( Σ_t q_total(t) ) · 0.6 / 3600,   CO2 (kg) = 2.31 · Fuel,   D̄ = mean over completed vehicles of w_v,   Δ_vs = 100 · (1 − D̄_advanced / D̄_controller)      (11)

The fuel figure assumes 0.6 L per hour for an idling vehicle; both constants are assumptions of the simulator and make fuel and CO2 proportional to total queued vehicle-seconds. The light-bar score used by the vision rule is s = min(1, 2·min(r, b)/(r + b) · min(1, (r + b)/120)), where r and b are the numbers of red and blue pixels in the cluster; a cluster must have at least 12 pixels of each colour, and a vehicle box is relabelled ambulance when s > 0.25 for YOLO boxes (> 0.2 for the classical detector).

### 1.5.5 A worked example

Take junction J in the north-south phase with green age τ = 20 s. The north approach holds four cars and one bus (W = 4 + 3 = 7), the forecaster predicts 10 vehicles against a queue of 5 (0.7 · 5 = 3.5), vehicles worth 2 PCU are inbound, the head vehicle has waited 24 s (24/12 = 2) and the receiving queue is not full, so P_N = 7 + 3.5 + 2 + 2 = 14.5. The south approach holds two cars and a truck (W = 4), its forecast equals its queue, nothing is inbound and the head has waited 12 s (1), so P_S = 5. The phase pressure is Π_NS = 19.5. If the east-west phase has Π_EW = 47, then 47 > 2 · 19.5 + 4 = 43 and, because τ ≥ 13, the junction starts its 3-second clearance and switches. With Π_EW = 40 it would keep the green; plain adaptive control, without the forecast, inbound and fairness terms, would have seen Π_NS = 11 and switched at any Π_EW above 26.

### 1.5.6 The pressure code

The pressure of a phase is a few lines in smart_traffic/sim.py; it is reproduced because the whole of the "advanced" idea is in it.

def _pressure(self, j, phase, adv):

total = 0.0

for a, name in enumerate(APPROACHES):

if PHASE_OF[name] != phase:

continue

qq = self.q[j][a]

w = self.wq(j, a)

if adv:

w += 0.7 * max(float(self.pred[j, a]) - len(qq), 0.0)     # ML forecast

w += float(self._inbound[j, a])                           # arriving platoon

if qq:

w += qq[0].wait / 12.0                                # fairness

nh = self._next_hop_of_straight(j, a)

if nh is not None and len(self.q[nh[0]][nh[1]]) >= CAP - 4:

w *= 0.3                                              # downstream full

total += w

return total

## 1.6 Models and configuration

### 1.6.1 Queue forecaster

Table 4: Forecaster configuration.

| Item | Value |
| --- | --- |
| Model | scikit-learn 1.5.0 GradientBoostingRegressor: 160 trees, max depth 4, learning rate 0.08, subsample 0.9, random_state 0 |
| Baselines | RandomForestRegressor (80 trees, depth 10, random_state 0); persistence (the current queue); ridge regression in the extra study |
| Training data | 26,988 rows from six simulated 1,800 s runs (2 x 2 and 3 x 3 alternating, demand 0.7 / 1.0 / 1.3 / 1.6, start hours 7, 8, 12, 17), snapshots every 10 s from t = 40 s, target the queue 30 s later |
| Split | Random 75 % / 25 % with random_state 1 (the application); five further random splits and a leave-one-simulation-out study in run_experiments.py |
| Fit time | 5.9 s on the CPU; inference for 16 rows 0.18 ms |
| Use | Called every 10 simulated seconds by the advanced controller; output clipped to 0-40 |

As the application trains it, the model scores MAE 1.644 vehicles and R² 0.961 on the held-out quarter against 2.535 and 0.904 for persistence. Because the quarter is drawn at random from snapshots that are ten seconds apart, rows from the same run appear on both sides of the split; Chapter 8 repeats the evaluation with whole runs held out and reports that the result is much weaker.

### 1.6.2 Vision layer

Table 5: Vision configuration.

| Item | Value |
| --- | --- |
| Vehicle detector | Ultralytics 8.4.186 yolov8n.pt (COCO, not fine-tuned); classes car, motorcycle (bike), bus, truck; confidence threshold 0.3 (adjustable) |
| Light-bar rule | HSV: red H 0-8 or 172-180, blue H 100-130, S ≥ 120, V ≥ 190; 3 x 3 elliptical opening; dilation of 5 % of the shorter image side; connected components of the red-and-blue intersection; ≥ 12 pixels of each colour |
| Ambulance flag | A YOLO box whose upper region (from 0.3 box heights above its top to 0.6 below) contains a cluster with score > 0.25 |
| Classical detector | For synthetic overhead scenes: background and lane-paint removal, morphological clean-up, contour size classes (bike < 900 px area, car < 95 px long side, truck < 130 px, else bus) |
| Density report | PCU / 14 capacity; four levels; recommended green 12 s with an ambulance, else 12 + 3.2 PCU clamped to 15-60 s |
| Training | None: the project trains no vision model |

### 1.6.3 Experiment protocol

scripts/run_experiments.py fixes the protocol so that the numbers can be reproduced: controllers are compared on seeds 40 to 44 for 1,800 simulated seconds at demand x1.2 starting at 08:00, on both grids; the demand sweep uses five levels from x0.6 to x1.8 with three seeds and 1,200 s; the ambulance-heavy test generates one ambulance per 60 s on average for 3,600 s; the ablation removes one mechanism at a time; the forecaster study repeats the application's split five times and holds out each of the six training runs in turn. Results are given as mean and sample standard deviation over seeds. The first three seeds are the ones used by the Benchmark screen, so the screen reproduces a subset of the tables.

# 2. NOVELTY AGAINST THE BASE PAPER

## 2.1 What the base paper contributes

The base paper [1] adds a supervisory emergency-priority layer to an already optimised traffic controller. A single-class YOLO26m detector trained on 3,130 annotated images finds the roof light-bar of an emergency vehicle (precision 0.73, recall 0.69, mAP50 0.73, 11-14 ms per frame on a GPU); a confidence threshold of 0.5 and three consecutive detections trigger a priority arbitrator that ends the current phase after a yellow interval, forces green on the ambulance approach and restores the optimised plan afterwards; the whole is validated in SUMO on one modelled roundabout in Rabat, where the ambulance travel time falls from 70 s (GA-SGD) and 64 s (fixed-time) to 56 s and the total waiting time to 19,887 s. The paper treats detection and priority at a single junction and leaves multi-junction coordination, congestion forecasting and violation handling out of scope.

## 2.2 What is new in FlowSense

•  One working pipeline. The batch objective is a single chain - detect, forecast, adapt, prioritise, coordinate, enforce - and FlowSense implements every link in one product with a console, an API and a built-in validation harness, where the base paper reports a detector and a simulation study.

•  A coordinated advanced controller. The pressure of each phase combines passenger-car-equivalent queues, a 30-second machine-learning forecast of the build-up, vehicles arriving from the upstream junction within 8 s, a fairness term for the head-of-queue wait, and a spill-back damping factor when the downstream approach is nearly full; an ambulance is looked for 15 s upstream so that the junctions along its route turn green before it arrives (a green wave) instead of pre-empting one junction at a time.

•  Like-for-like benchmarking. The four controllers share the same random demand and the same ambulances, so differences are attributable to control; the benchmark, a demand sweep, an emergency-heavy test and an ablation are all reproducible from scripts/run_experiments.py.

•  Honest validation beyond the simulator. The vision layer was tested on 293 real dashcam frames and 600 real ambulance and non-ambulance photographs, and the forecaster by leaving whole simulation runs out; both exposed weaknesses that the product's own screens do not show.

## 2.3 What is standard

Several parts are established techniques used as they are: YOLOv8n with COCO weights [22], [21], passenger-car-equivalent weights [31], queue-based actuated control in the tradition of [25] and [26], gradient-boosted regression [28] and random forests [29] through scikit-learn [30], and the idea of pre-empting a signal for an emergency vehicle [9], [10]. The simulator, the advanced pressure rule and the integration are the project's own work; none of the learning components was invented here.

## 2.4 What is weaker than the base paper

•  No trained emergency detector. The base paper trains a dedicated light-bar detector; FlowSense uses an HSV colour rule on top of a stock detector, which recalled 1.3 % of 300 real ambulance photographs (Chapter 8).

•  No SUMO. The base paper validates in SUMO against a real intersection; FlowSense validates in a purpose-built simulator whose parameters are assumptions, so absolute numbers are not comparable.

•  Ground-truth emergency knowledge. In the simulation the controllers know where the ambulances are; the vision layer is not closed-loop with the controllers, so the base paper's temporal-confirmation arbitrator (three consecutive detections) is not implemented.

•  A small gain over plain adaptive control. The advanced controller improves on plain queue-actuated control by 4.3 % (2x2) and 5.3 % (3x3), within about one standard deviation, so its novelty is better supported for ambulances than for general traffic.

# 3. KEY CONTRIBUTIONS

The contributions of the project, stated at the strength the evidence supports, are:

1.  An integrated smart-traffic pipeline, FlowSense, that combines YOLOv8n vehicle detection with passenger-car-equivalent density, an HSV light-bar emergency rule, a 30-second queue forecaster, four signal controllers, a violation log and a FastAPI + React console on ports 8310 and 5310.

2.  A like-for-like simulation harness (the FlowSense simulator) for 2x2 and 3x3 junction grids in which four controllers - fixed-time, adaptive, a base-paper-style emergency-aware controller and the advanced controller - face identical seeded demand and identical ambulances.

3.  An advanced coordinated controller whose pressure rule adds forecast, inbound-platoon, fairness and spill-back terms and an upstream emergency green wave. It reduces average delay by 34.8 % (2x2) and 30.5 % (3x3) against fixed-time and ambulance delay by 71.5 % and 86.7 %.

4.  A 30-second queue forecaster (gradient boosting on 11 features, 26,988 simulated samples) that beats persistence by about a third on a random split (MAE 1.64 against 2.57) and whose limits - failure on an unseen congested run, about 3 s of closed-loop benefit - were measured rather than assumed.

5.  An honest evaluation: five-seed means with standard deviations, a demand sweep, an emergency-heavy test, an ablation, leave-one-simulation-out validation, and tests of the vision layer on 293 real frames and 600 real photographs, which show that the synthetic vision results do not transfer.

6.  A reproducible, tested software artefact: 38 passing unit and API tests, a 33-check end-to-end smoke test, a one-click run.bat, and scripts that regenerate every number in the report from JSON.