SAFEGRID-AI: AN EXPLAINABLE MULTI-AGENT SYSTEM FOR DYNAMIC POWER ROUTING

Methodology, Novelty and Key Contributions

Department of Computer Science and Engineering, TKR College of Engineering and Technology (Autonomous)

Batch A13 (CSE-A)  ·  Academic year [ACADEMIC YEAR]

Table 1: Project team

| S.No. | Student name | Roll number |
| --- | --- | --- |
| 1 | DISHA KAUSHAL | 23K91A0563 |
| 2 | BOLLAM RAKESH | 23K91A0542 |
| 3 | CHINNOLLA RAVINDHAR | 23K91A0554 |
| 4 | AFFAN MOHAMMED RAMY | 23K91A0505 |

Project guide: CHANDRASHEKAR

Chapter and section numbers quoted in the text (for example Section 6.6 or Chapter 8) refer to the full project report, 17.Final Document. Figures and tables in this document are numbered from 1.

# 1. OVERVIEW OF THE METHODOLOGY

This document describes how the system works from end to end and what is new in it. Part 1 gives the pipeline, the data, the algorithms and the reasons they were chosen; Part 2 sets the work against the base paper and separates what is new from what is standard; Part 3 lists the contributions.

## 6.1 Overall Methodology

The method has four ingredients, each answering one sentence of the problem statement. A simulated grid supplies physics: a pandapower [11] AC power flow on a three-node low-voltage feeder with seeded solar, load and price profiles. Agents supply decisions: each node has an agent that perceives a structured context and proposes one action per step. A Symbolic Safety Shield supplies safety: it approves, projects or rejects every proposal before anything is executed. An audit and explanation layer supplies accountability: every decision carries a structured explanation and every intervention is appended to a hash-chained log.

The evaluation method is an ablation. The same seven scenarios and seeds are run under three controllers: a non-LLM rule baseline, the agents with their proposals executed as requested, and the full SafeGrid-AI system with the shield between the agents and the grid. Because scenario and seed are identical, any difference in outcome is due to the controller or the safety layer.

The architecture follows the safe-learning literature. Shielding places a verified component between a learning agent and its environment and lets it override unsafe actions [12]; projecting an action onto the set of safe actions is the idea of the safety layer of Dalal et al. [13]; and surveys organise methods by how they respect safety constraints [14]. SafeGrid-AI adds the language-model proposer, the operator-facing explanation and an audit trail, in the setting of the multi-agent power systems of [8] and the LLM-based grid control of Grid-Agent [2].

The engine computes the metrics at every step; their exact definitions matter for reading Chapter 8:

Table 2: Evaluation metrics as computed by the engine

| Metric | Definition |
| --- | --- |
| Violations caused by actions | Limit violations in the executed state that are new, or worse, than in the grid state with every agent idle (the attribution rule of Section 6.5): what the shield is designed to prevent. |
| Steps with violations caused | Steps in which at least one such violation occurred. |
| Violations the raw proposals would cause | Counterfactual: violations of a power flow in which every raw proposal of the step is applied together. |
| Steps with pre-existing stress | Steps in which the grid violates a limit even with every agent idle. |
| Steps with any violation | Steps in which the executed state violates any limit, whatever the cause. |
| Voltage violation steps, line overload events | Steps with a bus outside 0.95-1.05 pu; line-steps above 100 % loading. |
| Energy cost | Import energy times price minus export energy times 60 % of the price, summed over the run (INR, illustrative tariff). |
| Renewable utilisation | 1 minus curtailed energy over available PV energy; n/a when there is no PV (line_congestion runs at 18:00, after sunset). |
| Battery utilisation | Charged plus discharged energy over the total battery capacity of the three nodes (80 kWh). |
| Intervention rate | (Projected + rejected proposals) over all proposals. |
| Fallback rate, decision latency | Share of decisions made by the fallback policy; mean time the provider took to answer. |
| Step time | Wall-clock time of one engine step (agents, shield, trial power flows, commit). |

## 6.2 Simulated Data and Scenarios

There is no external dataset in this project. The inputs of every run are generated from a seed, so any run can be reproduced exactly. No trained model exists, so no accuracy, precision, recall or F1-score is reported anywhere in this report.

For a time of day h in hours, solar output at a node is its peak power times a daily shape and a seeded cloud factor, load is its base load times a household shape with 5 % seeded noise, and price follows a time-of-use tariff:

sun(h) = sin(π (h − 6) / 12)^1.2 for 6 ≤ h ≤ 18, otherwise 0;     PV(h) = kWp · pv_scale · sun(h) · cloud      (1)

shape(h) = 0.55 + 0.35 · exp(−((h − 7.5) / 1.5)²) + 0.75 · exp(−((h − 19.5) / 2)²);     load(h) = base · load_scale · shape(h) · (1 + 0.05 ε)      (2)

The cloud factor is 0.88 plus 0.12 times a normalised five-point moving average of seeded Gaussian noise, clipped to 0.45-1.0. The tariff is 3.8 INR/kWh before 06:00, 5.2 until 10:00, 4.6 until 17:00, 9.0 until 22:00 and 5.0 afterwards; it is illustrative, not taken from a utility. Exported energy is credited at 60 % of the price of the same step. The time step is 15 simulated minutes.

Seven scenarios were defined. A scenario fixes the start time, length, scale factors for load, solar and price, the initial states of charge, the weakness of the grid, and a list of faults: intentionally unsafe or malformed proposals that replace an agent's answer for a range of steps. Faults apply to the two agent arms only, because the rule baseline has no model that could misbehave.

Table 3: The seven scenarios

| Scenario | Start | Steps | Settings | What it exercises |
| --- | --- | --- | --- | --- |
| normal | 09:00 | 32 | - | Balanced day; the mock provider occasionally oversizes a request (unsafe rate 0.15) |
| high_solar | 10:00 | 24 | load x 0.6, PV x 1.5; start SOC 0.35 / 0.30 | Surplus PV must be stored, exported or curtailed (unsafe rate 0.25) |
| peak_demand | 17:30 | 24 | load x 2.0, price x 1.2; start SOC 0.70 / 0.35 / 0.70 | Evening peak with heavy household and EV demand (unsafe rate 0.25) |
| battery_stress | 11:00 | 16 | start SOC 0.93 (node_1) and 0.13 (node_3) | Three injected faults: charge 12 kW, discharge 30 kW, discharge 8 kW beyond battery limits |
| line_congestion | 18:00 | 16 | line L0_2 limited to 0.009 kA; EV start SOC 0.30 | Injected fault: rated 7.4 kW EV charging in steps 1-8 overloads the weak feeder |
| voltage_violation | 10:00 | 16 | PV x 0.5, lines x 2.0 long, upstream 1.02 pu | Two injected faults: battery discharge (10 kW and 5 kW) at the feeder end pushes voltage up |
| unsafe_ai_action | 12:00 | 12 | - | Five injected faults: export 1000 kW, import 900 kW, discharge 50 kW, curtail 100 kW and one malformed output |

Note on run length. The strategy comparison of Chapter 8 ran every scenario for 24 steps (6 simulated hours), longer than the native length of the four shortest scenarios. Their injected faults all lie in the first steps and are included; the later steps are ordinary operation under the same scale factors.

## 6.3 Perception: Building the Agent Context

Each step, every agent receives a structured JSON context built by BaseAgent.perceive: the identifiers, step and time, the node's load and solar power, the battery (availability, capacity, state of charge, ratings, state-of-charge window), the price and its daily statistics, a weather summary, the net exchange, state of charge and voltage of the neighbouring nodes, the grid constraints (voltage band, loading limit, node import and export limits, local voltage, worst line and transformer loading), and the agent's own previous action and its result. The EV agent adds the vehicle's connection state, required and critical state of charge, and whether vehicle-to-grid is enabled.

The context contains numbers and fixed labels only, and the system prompt of the API provider tells the model to treat every value as data, never as an instruction. The no-action power flow is computed before the agents are asked, so all agents see the same starting state.

## 6.4 Algorithm Selection and Justification

Why a proposer and a separate deterministic shield. A language model can read a rich context and propose a sensible action, but its output is probabilistic and can be wrong in ways that are costly on a physical network; ReAct-style agents interleave reasoning and acting without any guarantee about the acting [16], and the safety literature lists unsafe exploration and distributional shift as separate problems [15]. A deterministic checker after the proposer makes the safety property independent of the proposer and testable: the shield can be run with thousands of random or hostile actions without a model.

Why projection instead of rejection only. Rejecting every unsafe request wastes the useful part of a request that is only too large. The shield therefore executes the largest safe scale of the action: if a request of 4 kW exceeds the safe 2 kW (a hypothetical case), 2 kW is executed. The mock provider's oversized requests are of this kind.

Why bisection. Scaling an action by λ in [0, 1] moves its effect from none to the full request. For the limits considered, the safe region in λ is an interval that contains 0, because the no-action state is the reference. Bisection finds its boundary with a fixed number of evaluations and needs no gradients of the power flow: at most 40 cheap local evaluations, 14 power-flow evaluations and four re-verifications.

Why sequential, cumulative validation. Three agents may each propose something safe alone and unsafe together, such as two prosumers on the same feeder exporting at once. The shield therefore validates proposals one by one, each seeing the set-points already accepted in the step. The coordinator puts actions that can only help (idle, curtail, shift load) first and rotates the rest with the step number.

Why attribution to the action. If the shield rejected whatever coexists with a violation, a grid already outside its limits with every agent idle would make every action look unsafe. The shield instead blames an action only for violations that are new or that it makes worse than the no-action state, which keeps agents useful during stress and states exactly what the guarantee is.

Why a hash chain. Safety events are the evidence that the shield acted. A SHA-256 chain over the events, as specified in FIPS 180-4 [20], makes tampering detectable without any external service; it is a cheap integrity check, not a replacement for access control.

Why a mock provider. Experiments must be reproducible and free of cost and network dependence. The mock provider is deterministic for a given seed, agent and step; it is not a language model, and the report says so wherever results are given.

## 6.5 Mathematical Formulation

Let V_b be the voltage magnitude (per unit) at low-voltage bus b, I_l and I_l^max the current and rated current of line l, and T the transformer loading. The grid constraints applied to every candidate state are:

0.95 ≤ V_b ≤ 1.05  for every bus b;     100 · I_l / I_l^max ≤ 100  for every line l;     T ≤ 100 %;     −200 kW ≤ P_ext ≤ 200 kW      (3)

The voltage band is narrower than the ±10 % band that EN 50160 [19] allows for public low-voltage supply, so a violation of the shield's limit is an early warning, not an incident. All limits are configuration values.

For a node with battery capacity C, charge and discharge efficiencies η_c and η_d (both 0.95), state of charge s, window [s_min, s_max] and time step Δt (hours), the largest battery power inside the window, and the state update on commit, are:

P_charge ≤ min( P_charge^max , (s_max − s) · C / (Δt · η_c) );     P_discharge ≤ min( P_discharge^max , (s − s_min) · C · η_d / Δt )      (4)

s' = s + P · Δt · η_c / C  (charging, P > 0);     s' = s + P · Δt / (η_d · C)  (discharging, P < 0)      (5)

The physical effect of an action on a node is a set-point (storage power S, curtailment c, shifted load d). Import and export actions are targets for the node's net exchange E; with the neutral exchange E0 = load − PV, the storage covers the difference:

E = (load − d) + S − (PV − c);     import target P: S = P − E0;     export target P: S = −P − E0      (6)

The node limits are E ≤ import limit and −E ≤ export limit. After each power flow the shield also checks that the network's power balance closes:

r = P_ext + P_gen − P_load − P_stor − P_loss;     |r| ≤ 0.05 kW      (7)

Projection. For a request with set-point s the shield looks for the largest scale factor that is safe:

λ* = max { λ ∈ [0, 1] : safe(λ · s) },     safe(x) = no new or worse violation when x is applied      (8)

If safe(s) holds, λ* = 1 and the action is approved. Otherwise bisection is used: with lo = 0 and hi = 1, the midpoint replaces lo when it is safe and hi otherwise; 40 halvings are used for the local limits (resolution 2^−40) and 14 for power-flow limits (about 6 × 10^−5). For import and export actions the power after projection is |E0 + λ (E_req − E0)|, so the scaling moves the node's exchange between its neutral value and the request. The scaled power is rounded down to six places and checked again, up to four times with λ reduced by 0.1 % each time, so the executed action is exactly the verified action. A projection below 0.05 kW, or one that would turn an import into an export or the reverse, is rejected.

Attribution. Let V(x) be the set of violations of a state x and ex(v) the excess of a violation v beyond its limit. For the state after an action a and the no-action state 0, the violations blamed on the action are:

V_caused(a) = { v ∈ V(after a) : v ∉ V(0)  or  ex(v) > ex_0(v) + 10^−4 }      (9)

Audit chain. With h_0 a string of 64 zeros and for the i-th event with identifier id, run r, step k, simulated time t and canonical JSON payload p:

h_i = SHA-256( h_{i−1} ‖ canon([id_i, r, k, t_i]) ‖ canon(p_i) )      (10)

## 6.6 Agents, Providers and Fallback

Three agents control the three nodes: prosumer_01 (residential prosumer, node_1, 6 kWp, 10 kWh), ev_01 (EV and storage, node_2, 40 kWh, 7.4 kW, charge only by default) and renewable_01 (renewable prosumer, node_3, 20 kWp, 30 kWh). They share the BaseAgent class; the subclasses differ only in the extra context they add. A proposal is an EnergyAction: one of seven action types (charge, discharge, import, export, curtail, shift load, idle), a power in kW, a duration, an optional target node, up to eight reason codes and a confidence.

The action schema is strict: unknown fields are forbidden; power must be finite and between 0 and 1,000,000 kW; the duration lies between 1 and 1,440 minutes; reason codes must match [A-Z0-9_]{1,48}. An answer that fails the schema becomes a provider error and never reaches the shield as an action.

Table 4: LLM providers

| Provider | Behaviour | Used for |
| --- | --- | --- |
| mock | Deterministic stand-in for a language model: the transparent cost-aware policy plus seeded imperfections (oversized or full-rating requests) | All measured results |
| api | HTTP tool-calling provider (Anthropic or OpenAI wire format); the answer must be one propose_energy_action call that validates against EnergyAction | Implemented, not run live |
| baseline | Non-LLM time-of-use rules that ignore solar surplus and grid stress | Rule baseline arm |
| failing | Always raises an unavailability error to exercise the fallback path | Tests |
| fault-injecting wrapper | Replaces chosen agent/step proposals with unsafe or malformed ones (scenario faults) | Agent arms |

Fallback. BaseAgent.decide waits for the provider with a timeout (20 s by default). If the provider is unavailable, times out, or returns an invalid answer, the agent uses the deterministic cost-aware policy and labels the decision fallback with the reason; it is shielded like any other. In the experiment, the fallback rate was 0.2 % of decisions, all from the injected malformed output at step 6 of the unsafe_ai_action scenario.

The deterministic policy. The cost-aware policy is a transparent set of rules, not an optimiser, and no optimality is claimed: it absorbs surplus solar into the battery, curtails when the voltage is high and the battery is full, discharges to support the grid or at high prices, charges the EV towards its target when the price allows, and otherwise idles. It underlies the mock provider and the fallback. The rule baseline is simpler and ignores solar and grid state.

## 6.7 Symbolic Safety Shield and Projection

SymbolicSafetyShield.evaluate receives one EnergyAction, a read-only view of the node, a function that runs a trial power flow for a candidate set-point (including the set-points already accepted in the step), and the current grid state. Its ten checks run in a fixed order, grouped by the violation codes they can raise; the dashboard shows the same ten names.

Table 5: The ten shield checks (25 violation codes)

| # | Check | Codes | What it tests |
| --- | --- | --- | --- |
| 1 | STRUCTURE | 7 | Unknown or mismatched agent, node or target; non-finite or negative power; invalid node state; malformed type |
| 2 | POWER_LIMITS | 1 | Power larger than 5 times the node's reference power is implausible |
| 3 | BATTERY | 4 | Battery missing or unavailable; charge or discharge rating; state-of-charge window |
| 4 | NODE_LIMITS | 3 | Node import limit, node export limit, shiftable share of load |
| 5 | POWER_BALANCE | 3 | Curtailment above generation; unreachable import/export direction; balance residual above 0.05 kW |
| 6 | POWER_FLOW | 1 | The AC power flow must converge |
| 7 | VOLTAGE | 2 | Every low-voltage bus within 0.95-1.05 pu |
| 8 | LINE_LOADING | 1 | Every line at or below 100 % of its rated current |
| 9 | TRANSFORMER | 1 | Transformer at or below 100 % loading |
| 10 | GRID_LIMITS | 2 | Import and export at the external grid at or below 200 kW |

The procedure is: (1) structural checks; a violation rejects the action. (2) An idle or about-zero action is approved and recorded. (3) The action is converted to a set-point; the local checks and, if they pass, a trial power flow are run; only new or worse violations count. (4) With none, the action is approved, with its duration aligned to the time step if necessary. (5) Otherwise the largest safe scale is found by bisection, first for the local limits and then for the power-flow limits, and the rounded action is re-verified. (6) The action is rejected if nothing safe remains, if the safe part is under 0.05 kW, or if the direction of an import or export target cannot be kept. (7) Otherwise the scaled action is returned as PROJECTED with the corrections made (requested and executed power, and the violation codes that caused the scaling).

The returned ShieldDecision keeps the original request, the validated action, every violation with its code, component, requested and allowed values, the corrections, the result of each of the ten checks (passed, failed, or not evaluated because an earlier check rejected the action), and the grid summary before and after. The original request is never overwritten, so the audit trail always shows what the agent asked for.

Failures are conservative: a power flow that does not converge raises POWER_FLOW_FAILED and the action is rejected. The shield module imports nothing from the model, agent, simulation or grid packages and receives only the structured fields of the action.

## 6.8 Coordinator

The coordinator never edits an action. It detects three kinds of conflict and fixes the validation order. PEER_MISMATCH is raised when some agents propose to export while others propose to import and the totals differ by more than 0.5 kW; the residual goes to the external grid. TARGET_CONTENTION is raised when several proposals name the same target node. SHARED_FEEDER is raised when node_1 and node_3, whose feeder shares line L0_1, both push power the same way by more than 0.5 kW. Conflicts are published as events and counted: 241 were detected in the 21 shielded runs. The order puts relief actions first and rotates the rest by step.

## 6.9 Explanations

Every decision has a structured explanation, built from the context and the decision, not generated by a model: the decision and a readable label, the power, the reason codes in plain sentences, the confidence, the values the agent saw (solar, load, state of charge, price, local voltage, worst line loading), the decision source (mock, llm, baseline, fallback, injected) and, once the shield has answered, the safety result, the violations and the executed power.

No chain-of-thought is requested or stored. The batch abstract speaks of explainable reasoning; the implementation deliberately shows inputs, decision factors, the action and the safety result instead of free-text reasoning. A model's stated reasoning is not always a faithful account of why it answered as it did [17], whereas the inputs and the shield's checks can be verified. A test (test_explanation_is_structured_and_has_no_chain_of_thought) checks that the explanation has no chain-of-thought or reasoning field.

# 2. NOVELTY AGAINST THE BASE PAPER

## 2.1 Positioning

Three works frame the novelty claim. The base paper [1] designs a rule-based multi-agent energy management system for a DC microgrid, regulates the bus voltage at 800 V in eight Simulink scenarios, and lists scalability, software integration, cybersecurity and the lack of experimental validation as open issues. Grid-Agent [2] uses a language-model planning agent and a validation agent that runs candidate actions in a sandbox with rollback on IEEE and CIGRE networks; its best configurations report a 100 % success rate and weaker ones 63 % and 44 %. Picault and Goubet [3] argue in a position paper for Model Context Protocol access to grid simulators under human oversight and set requirements such as traceability and human validation, without quantitative results.

SafeGrid-AI sits between these: it keeps the multi-agent structure of the first, the language-model proposer of the second and the traceability and oversight concern of the third, and it adds an independent safety layer with an explanation and audit interface. The claim is about the combination and the evaluation, not about a new learning algorithm.

## 2.2 What Is New

Table 6: Novelty compared with the three framing papers

| Element | Base paper [1] | Grid-Agent [2] | MCP position paper [3] | SafeGrid-AI |
| --- | --- | --- | --- | --- |
| Decision makers | Rule-based agents | LLM planner | LLM agents via MCP tools | Pluggable provider (mock measured), deterministic fallback |
| Safety mechanism | Control keeps bus voltage regulated | Sandboxed trial with rollback | Human validation (proposed) | Deterministic shield: approve, project or reject before execution |
| Action modification | - | Revise plan and retry | - | Largest safe scale found by bisection, original request preserved |
| Attribution of violations | - | - | - | New-or-worse than the idle grid state |
| Explanation | - | - | Traceability as a requirement | Structured explanation per decision, no stored chain-of-thought |
| Audit | - | - | Traceability as a requirement | SHA-256 hash-chained safety log, verified in 21 of 21 runs |
| Evidence | 8 Simulink scenarios | IEEE/CIGRE benchmarks | None (position paper) | 63 runs, 7 scenarios, 3 strategies, 3 seeds, counterfactual of raw proposals |

Five elements are new in this project, in a modest sense of the word:

1.  A verdict with three values. A veto-only validator forces the agent to guess again. The shield returns the largest safe fraction of the request by bisection, so the agent's intent survives at a smaller size; of 1,512 proposals, 237 were handled this way. Projection of an unsafe action onto a safe set is a known idea in safe learning [13], applied here to a grid feeder with a full power flow in the loop.

2.  Attribution against the idle baseline. A violation is blamed on an action only if it is new or worse than the same step with every agent idle. This lets the evaluation separate the harm an action does from stress the scenario already contains, and it stops the shield from blocking an action that improves an already violated grid.

3.  Enforced independence. The shield imports nothing from the language-model, agent, simulation or grid packages, and a test fails if it ever does. The deterministic layer therefore cannot be influenced by the layer it supervises.

4.  Explanations from structure. Decisions are explained from the inputs, decision factors and safety result. Free-text model rationales are not reliable accounts of behaviour [17], so none is requested or stored.

5.  A counterfactual evaluation. Besides measuring what happened, every step also computes what the raw proposals would have done without the shield, so the benefit is measured and not assumed: 192 violations in 145 steps for the shielded arm's proposals.

## 2.3 What Is Standard

Most of the system is standard engineering, and the report does not claim otherwise: multi-agent decomposition of a power system [8], [18]; AC power flow with pandapower [11]; a closed action schema and sandboxing of model output; bisection for a monotone constraint; SHA-256 hash chaining [20]; a REST and WebSocket API; and a React dashboard. The language-model interface follows the common pattern of a tool call returning structured arguments [16]. Shielding as a concept comes from safe reinforcement learning [12], [14], and the concern that learning systems act unsafely goes back to the safety problems catalogued by Amodei et al. [15].

## 2.4 Limits of the Novelty Claim

The claim is bounded by the evidence: the shield was evaluated on a three-node simulated feeder with a mock language model, so it has not been shown to be better than the sandbox-and-rollback approach of Grid-Agent, which was evaluated on other networks and with real models. No numeric comparison with any of the three papers is possible, because the systems, benchmarks and metrics differ.

# 3. KEY CONTRIBUTIONS

The contributions of the project, stated as things that now exist and were checked:

1.  A neuro-symbolic architecture in running code. A proposer, which may be a language model, and a deterministic validator separated by a closed EnergyAction schema and by enforced module independence, with a coordinator in between (Chapter 5, Chapter 6).

2.  A Symbolic Safety Shield with projection. Ten ordered checks (structure, power limits, battery, node limits, power balance, power flow, voltage, line loading, transformer, grid limits), 25 violation codes and bisection projection with re-verification of the rounded action. In 21 shielded runs it handled 1,512 proposals and caused no violation in 504 steps.

3.  A violation-attribution rule and an evaluation built on it. Three arms (rule baseline, agents without shield, SafeGrid-AI), seven scenarios, three seeds, per-scenario tables and the counterfactual count of what raw proposals would have caused.

4.  Explainability and audit by design. A structured explanation for every decision, a hash-chained safety log verified in 21 of 21 runs, and a dry-run tool that lets an operator test any action against the shield without applying it.

5.  An operator console. A React application with live run control, single-line diagram, agent monitor, shield view, power-flow charts, scenario runner and explainability view, tested in a browser at three widths and two themes with no console errors.

6.  An honest record. The documents state what the measurements do not show: a mock model, no violation-resolution rate, three seeds, a model-relative guarantee, two failing tests and one hanging test, and no user study.

Reuse by others is straightforward: any controller that can return an EnergyAction can be placed behind the provider interface and measured on the same scenarios, with or without the shield; the 63-run result file is kept at docs/results/experiment_3seeds_24steps.json.

Table 7: Contributions and how each was checked

| Contribution | Where it lives | How it was checked |
| --- | --- | --- |
| Proposer / validator separation | models/actions.py, safety/shield.py | test_malformed_actions_fail_schema_validation; test_shield_does_not_import_llm_or_agents |
| Shield with projection | safety/validator.py, projector.py, shield.py | 22 shield tests; 504 shielded steps with 0 violations caused |
| Violation attribution | validator.new_violations, engine.step | test_preexisting_violation_does_not_block_a_relieving_action; experiment metrics |
| Coordinator | simulation/coordinator.py | test_coordinator_detects_peer_mismatch; 241 conflicts reported in 21 runs |
| Explanations | agents/explain.py | test_explanation_is_structured_and_has_no_chain_of_thought |
| Hash-chained log | safety/events.py | test_event_chain_is_tamper_evident; chain valid in 21 of 21 runs |
| Three-arm experiment | simulation/experiment.py | test_experiment_runs_real_simulations; 63 runs saved to JSON |
| Operator console | frontend/src | Browser walk at 360, 768 and 1440 px, light and dark; smoke test 20 of 20 |

What is not claimed as a contribution: a new learning algorithm, a new power-flow method, an evaluation of any real language model, or a safety proof. The contribution is a design, an implementation of it, and a measurement of what the design does inside the limits stated in the limitations document.